贯通项目 步骤二  单元：二输入与非门的版图与验证

Capstone Step 2  Cell: Layout and Verification of a Two-Input NAND Gate

微电子贯通项目 · 从晶体管到芯片 · 机器人关节电机控制芯片    Microelectronics capstone · From transistor to chip · A motor-control chip for a robot joint

整个项目为机器人关节设计一块电机控制芯片的数字部分：PWM 发生器控制电机电压，正交编码器计数器测量转过的角度。四步分别对应半导体器件物理、数字集成电路设计、硬件描述语言与集成电路验证、VLSI 物理设计四门课，最后交出一份符合 Tiny Tapeout 格式、时序、DRC、LVS 全部通过的芯片版图。

The project designs the digital part of a motor-control chip for a robot joint: a PWM generator sets the motor voltage and a quadrature encoder counter measures the angle turned. The four steps belong to four courses (device physics, digital IC design, HDL and verification, VLSI physical design) and end with a Tiny Tapeout-format layout that passes timing, DRC and LVS.

# 一、实验目的  1  Objectives

1. 写出二输入与非门的晶体管级网表：两个 NMOS 串联、两个 PMOS 并联。
Write the transistor netlist of a two-input NAND: two NMOS in series, two PMOS in parallel.

2. 在 Magic 里把四个已放好的晶体管连成与非门，通过设计规则检查（DRC）和版图与原理图一致性检查（LVS）。
Wire four placed transistors into a NAND gate in Magic and pass DRC and LVS.

3. 与 SKY130 标准单元 nand2_1 比较面积和管宽，理解标准单元库为什么这样设计。
Compare area and widths with the SKY130 standard cell nand2_1 and see why standard cells look the way they do.

# 二、实验原理  2  Principles

与非门 Y = ¬(A·B)：两个输入都为高时串联的 NMOS 都导通，输出被拉低；任一输入为低时对应的 PMOS 导通，输出为高。

NAND Y = ¬(A·B): with both inputs high the series NMOS conduct and pull the output low; with either input low its PMOS conducts and the output is high.

串联的两个 NMOS 相当于沟道长度加倍，下拉能力减半；为了和反相器一样快，NMOS 要加宽到约 2 倍。并联的 PMOS 最坏情况只有一个导通，宽度与反相器的 PMOS 相同即可。

Two NMOS in series act like a doubled channel length: half the pull-down strength. To match an inverter they are made about twice as wide. In the worst case only one parallel PMOS conducts, so it keeps the inverter's PMOS width.

版图规则（DRC）保证工厂能做出来：最小线宽、间距、接触孔的包围等；LVS 从版图提取出电路，与原理图逐个器件、逐个节点比较，并核对器件尺寸。

Design rules (DRC) make the layout manufacturable: minimum widths, spacings, contact enclosures. LVS extracts the circuit from the layout and compares it device by device and node by node with the schematic, including sizes.

本实验的四个管子各带一圈衬底保护环、PMOS 各在自己的 n 阱里，第一层金属（metal1）会和栅极走线交叉，所以输出 OUT 要用第二层金属（metal2）经 via1 连接。标准单元则让串联管共用扩散区、PMOS 共用一个 n 阱，面积小一个数量级。

Here each transistor has its own guard ring and each PMOS its own n-well, and metal1 would cross the gate wires, so OUT runs on metal2 through via1. A standard cell shares diffusion between the series transistors and one n-well for both PMOS, and is an order of magnitude smaller.

# 三、实验环境与起始文件  3  Environment and starting files

本步在问渠微电子实验镜像中完成（网页远程桌面或命令行）。

This step runs in the WenQuest microelectronics lab image (web desktop or command line).

入口 Access: 问渠课程页“实验镜像”（网页远程桌面），或教师提供的镜像 wenquest/micro-lab；起始文件用 wqlab start 我的目录 复制 / the lab image from the WenQuest course page (web desktop) or the image wenquest/micro-lab; copy the starting files with wqlab start my_folder

| 项目 Item | 内容 Details |
| --- | --- |
| 工具 Tools | Magic（版图、DRC、提取）、Netgen（LVS）、Xschem（可选，画原理图） Magic (layout, DRC, extraction), Netgen (LVS), Xschem (optional, schematic) |
| 起始文件 Starting files | step2_nand2/nand2.mag（四个管子已放好）；nand2_schematic.spice（网表模板）；placement.json（各端子坐标） nand2.mag (four transistors placed); nand2_schematic.spice (netlist template); placement.json (terminal coordinates) |
| 管宽 Widths | Wn = 0.42 μm，Wp = 0.97 μm（可按原理二加宽 NMOS，需重新生成起始版图） Wn = 0.42 μm, Wp = 0.97 μm |
| 检查命令 Check | wqlab nand2 --check step2_nand2 --schematic step2_nand2/nand2_schematic.spice |

# 四、实验步骤  4  Procedure

1. 在 nand2_schematic.spice 里写出四个晶体管（端口顺序 A B OUT VDD VSS，串联中间节点叫 X）。
Write the four transistors in nand2_schematic.spice (ports A B OUT VDD VSS; call the series node X).

2. 用 magic -rcfile $MAGICRC step2_nand2/nand2.mag 打开版图，看清 N1、N2（下排）和 P1、P2（上排）的源、漏、栅端子（placement.json 有坐标）。
Open the layout with magic -rcfile $MAGICRC step2_nand2/nand2.mag; find the source, drain and gate of N1, N2 (bottom) and P1, P2 (top) (coordinates in placement.json).

3. 用 metal1 画：两条栅极竖条和引脚 A、B；N1 与 N2 之间的 X；底部 VSS、顶部 VDD 电源轨（压在保护环上，用 mcon 接到保护环），以及到各源/漏的短线。
Draw in metal1: the two gate strips and pins A, B; X between N1 and N2; the VSS rail at the bottom and VDD at the top (over the guard rings, joined by mcon), and the short wires to sources and drains.

4. 用 metal2 画 OUT，经 via1 连到 P1、P2 的一个端子和 N2 的另一个端子；给 A、B、OUT、VDD、VSS 加标签并设为端口。只保存顶层单元（File → Save）。
Draw OUT in metal2 through via1 to one terminal of P1, of P2 and of N2; label A, B, OUT, VDD, VSS as ports. Save only the top cell (File → Save).

5. 运行检查命令。DRC 不为 0 时按提示的规则修改（Magic 里 drc why 可指出位置）；LVS 不一致时看 lvs.out。通过后记录面积，填表 1、表 2。
Run the check. If DRC is not 0, fix the named rules (drc why in Magic shows where); if LVS fails, read lvs.out. When it passes, record the area in Tables 1 and 2.

# 五、数据记录  5  Data record

以下表格同时出现在实验报告模板中，请在报告里填写，并附上检查命令生成的 result.json。

The same tables are in the report template; fill them in there and attach the result.json from the check.

表 1  检查结果 Table 1  Check results

| 项目 Item | 结果 Result |
| --- | --- |
| DRC 错误数 DRC errors |  |
| LVS 一致 LVS match |  |
| 器件尺寸核对 Sizes checked |  |
| 版图宽 × 高 μm |  |
| 面积 μm² |  |

表 2  与标准单元比较 Table 2  Against the standard cell

| 项目 Item | 我的与非门 Mine | sky130_fd_sc_hd__nand2_1 | 比值 Ratio |
| --- | --- | --- | --- |
| 面积 μm² |  | 3.754 |  |
| NMOS 宽 μm |  | 0.65 |  |
| PMOS 宽 μm |  | 1.0 |  |
| 布线层 Routing layers |  | li1、metal1 |  |

# 六、注意事项  6  Notes

只保存顶层单元，不要用 writeall：重新保存的器件单元会被改写，下次打开时尺寸出错。
Save only the top cell, never writeall: re-saved transistor cells are rewritten and come back the wrong size.

metal1 最小宽度 0.14 μm、间距 0.14 μm；via1 连同包围至少 0.26 μm 见方，metal1、metal2 还要在一个方向多出 0.06 μm。
metal1 minimum width and spacing 0.14 μm; via1 with its enclosure at least 0.26 μm square, and metal1/metal2 extend 0.06 μm further in one direction.

参考结果：DRC 0、LVS 一致，面积约 55 μm²，是标准单元的约 14 倍。
Reference: DRC 0, LVS match, area about 55 μm², about 14 times the standard cell.

# 七、思考题  7  Questions

1. 标准单元 nand2_1 的 NMOS 比 PMOS 窄还是宽？和原理二的分析一致吗？为什么？
Is the NMOS in nand2_1 narrower or wider than its PMOS? Does that match principle 2? Why?

2. 你的版图面积是标准单元的十几倍，主要多在哪里？如果自己重画，怎样把面积减下来？
Your layout is more than ten times the standard cell. Where does the area go, and how would you shrink it?

# 八、与实际问题的联系  8  Connection to a real problem

机器人问题：编码器两路信号的“同时到”检测

Robot problem: Detecting that both encoder lines arrived

编码器计数器里要判断 A、B 两路同时为高。(1) 用与非门加反相器实现“与”，画出晶体管级电路。(2) 用第 1 章的 RC 模型估算：两个串联 NMOS 与单个 NMOS 相比，下拉电阻、下降延时各变为多少倍？(3) 要让下降延时与反相器相同，NMOS 应加宽到多少？与标准单元的实际管宽比较。

The encoder counter must detect that A and B are both high. (1) Build AND from a NAND and an inverter; draw the transistors. (2) With chapter 1's RC model, by how much do the pull-down resistance and fall delay change for two series NMOS versus one? (3) How wide must the NMOS be to match the inverter's fall delay? Compare with the standard cell.

请在实验报告“实际问题的建模与求解”一栏中按五步完成：① 实际问题 ② 建立模型（写出简化假设）③ 求解 ④ 用实验或仿真检验 ⑤ 指出模型在哪里失效、怎样修正。

In the report section “Modelling a real problem”, work through five steps: ① the problem ② the model (state your assumptions) ③ the solution ④ a check by experiment or simulation ⑤ where the model fails and how to improve it.

生活中的例子：两道门串联的走廊：两扇门都开着才走得过去（串联 = 与），而旁边两条并排的路走哪条都行（并联 = 或）。

Everyday example: A corridor with two doors in a row: you get through only if both are open (series = AND); two side-by-side paths let you through by either (parallel = OR).

# 九、评分  9  Grading

按贯通项目评分量规：原理 15、数据 20、分析 20、实际问题 20、规范 10、流片就绪度 15。本步的流片就绪度按下表由检查结果自动给分：

Graded with the capstone rubric: principles 15, data 20, analysis 20, real problem 20, presentation 10, tapeout readiness 15. This step's readiness points come from the check results:

| 检查项 Check | 分值 Points | 条件 Condition |
| --- | --- | --- |
| DRC 错误为 0 DRC errors 0 | 5 | drc_errors = 0 |
| LVS 一致（含尺寸）LVS match incl. sizes | 7 | lvs_match = true |
| 与标准单元比较完成 Comparison with the standard cell | 3 | area_ratio_vs_stdcell 有值 |