贯通项目 步骤一  器件：SKY130 MOSFET 特性与参数提取

Capstone Step 1  Device: SKY130 MOSFET Characteristics and Parameter Extraction

微电子贯通项目 · 从晶体管到芯片 · 机器人关节电机控制芯片    Microelectronics capstone · From transistor to chip · A motor-control chip for a robot joint

整个项目为机器人关节设计一块电机控制芯片的数字部分：PWM 发生器控制电机电压，正交编码器计数器测量转过的角度。四步分别对应半导体器件物理、数字集成电路设计、硬件描述语言与集成电路验证、VLSI 物理设计四门课，最后交出一份符合 Tiny Tapeout 格式、时序、DRC、LVS 全部通过的芯片版图。

The project designs the digital part of a motor-control chip for a robot joint: a PWM generator sets the motor voltage and a quadrature encoder counter measures the angle turned. The four steps belong to four courses (device physics, digital IC design, HDL and verification, VLSI physical design) and end with a Tiny Tapeout-format layout that passes timing, DRC and LVS.

# 一、实验目的  1  Objectives

1. 在开源工艺 SKY130 的晶体管模型上测出 NMOS、PMOS 的转移特性和输出特性。
Measure the transfer and output characteristics of SKY130 NMOS and PMOS transistors.

2. 用最大跨导外推法提取阈值电压，读出饱和电流、关态电流和亚阈值摆幅。
Extract the threshold voltage by maximum-transconductance extrapolation; read the saturation current, off current and subthreshold swing.

3. 比较提取值与数字集成电路设计第 1 章用的“有效参数”，理解短沟道器件和平方律模型的差别。
Compare the extracted values with the effective parameters of Digital IC Design chapter 1 and see how a short-channel device differs from the square law.

# 二、实验原理  2  Principles

平方律模型：饱和区 ID = ½k′(W/L)(VGS − VT)²，线性区 ID ≈ k′(W/L)(VGS − VT)VDS。线性区里 ID 随 VGS 近似直线增长，在跨导 gm = dID/dVGS 最大处作切线，与横轴交点减去 VDS/2 即阈值电压 VT。

Square law: saturation ID = ½k′(W/L)(VGS − VT)²; linear region ID ≈ k′(W/L)(VGS − VT)VDS. In the linear region ID grows almost linearly with VGS; the tangent at maximum gm = dID/dVGS meets the axis at VT + VDS/2.

亚阈值区电流随 VGS 指数变化：ID ∝ exp(VGS / nVth)。电流每增大 10 倍所需的 ΔVGS 叫亚阈值摆幅 S = n·Vth·ln10，室温下理想值约 60 mV/dec，实际器件更大。S 决定关态电流和开关比 ION/IOFF。

Below threshold the current is exponential: ID ∝ exp(VGS / nVth). The VGS change per decade of current is the subthreshold swing S = n·Vth·ln10, about 60 mV/dec at best at room temperature. S sets the off current and the ratio ION/IOFF.

短沟道器件（L = 0.15 μm）在饱和区出现速度饱和，电流随 (VGS − VT) 增长慢于平方。所以用平方律描述它时，要想同时对上饱和电流和传输特性，只能用比外推值小的“有效 VT”——第 1 章正是这样校准的。

A short-channel device (L = 0.15 μm) shows velocity saturation: its current grows more slowly than the square of (VGS − VT). A square-law description that matches both the saturation current and the transfer curve therefore needs an effective VT smaller than the extrapolated one — which is how chapter 1 was calibrated.

# 三、实验环境与起始文件  3  Environment and starting files

本步在问渠微电子实验镜像中完成（网页远程桌面或命令行）。

This step runs in the WenQuest microelectronics lab image (web desktop or command line).

入口 Access: 问渠课程页“实验镜像”（网页远程桌面），或教师提供的镜像 wenquest/micro-lab；起始文件用 wqlab start 我的目录 复制 / the lab image from the WenQuest course page (web desktop) or the image wenquest/micro-lab; copy the starting files with wqlab start my_folder

| 项目 Item | 内容 Details |
| --- | --- |
| 器件 Devices | sky130_fd_pr__nfet_01v8 / pfet_01v8，W = 0.42 μm，L = 0.15 μm W = 0.42 μm, L = 0.15 μm |
| 工艺角与温度 Corner, temperature | tt，27 °C tt, 27 °C |
| 仿真器 Simulator | 镜像里的 ngspice（也可在浏览器 ngspice 页试单个工作点） ngspice in the image (single points also in the browser ngspice page) |
| 起始文件 Starting file | step1_device/device_curves.py：扫描已写好，完成 extract() sweeps written; complete extract() |
| 检查命令 Check | wqlab device --script step1_device/device_curves.py --out step1_device/out |

# 四、实验步骤  4  Procedure

1. 运行检查命令，先看生成的 nmos.png、pmos.png 和 CSV 数据：转移特性（对数坐标）、输出特性。
Run the check command; look at nmos.png, pmos.png and the CSV files: transfer curves (log scale) and output curves.

2. 在 extract() 里完成 TODO 1：用相邻点差分算 gm，找最大 gm 处作切线，求 VT（记得减 VDS/2）。
In extract(), complete TODO 1: compute gm by differences, take the tangent at maximum gm and find VT (subtract VDS/2).

3. 完成 TODO 2、3：读出 IDSAT（|VGS| = |VDS| = 1.8 V）、IOFF（VGS = 0，|VDS| = 1.8 V），用 |VDS| = 1.8 V 的曲线在亚阈值区求摆幅 S（两条曲线算出的 S 略有不同，统一用这一条）。
Complete TODO 2 and 3: read IDSAT (|VGS| = |VDS| = 1.8 V) and IOFF (VGS = 0, |VDS| = 1.8 V); find the swing S below threshold on the |VDS| = 1.8 V curve (the two curves give slightly different S; use this one).

4. 再运行检查命令，把 out/params.json 的结果填入表 1；计算迁移率比 IDSAT,n / IDSAT,p。
Run the check again and copy out/params.json into Table 1; compute the ratio IDSAT,n / IDSAT,p.

5. 把提取的 VT 和第 1 章的有效 VT、k′ 比较，填表 2，并用平方律反推：若 VT 用提取值，k′ 要多大才能对上 IDSAT？
Compare your VT with chapter 1's effective VT and k′ (Table 2); with the square law, what k′ would match IDSAT if VT took your extracted value?

# 五、数据记录  5  Data record

以下表格同时出现在实验报告模板中，请在报告里填写，并附上检查命令生成的 result.json。

The same tables are in the report template; fill them in there and attach the result.json from the check.

表 1  提取的器件参数 Table 1  Extracted parameters

| 参数 Parameter | NMOS | PMOS | 单位 Unit |
| --- | --- | --- | --- |
| VT（外推） |  |  | V |
| IDSAT（每微米） |  |  | μA/μm |
| IOFF |  |  | A |
| S |  |  | mV/dec |
| ION/IOFF |  |  | — |

表 2  与第 1 章有效参数比较 Table 2  Against chapter 1's effective parameters

| 参数 Parameter | 提取 Extracted | 第 1 章 Chapter 1 | 说明 Comment |
| --- | --- | --- | --- |
| VT V |  | 0.55 |  |
| k′n μA/V² | （按提取 VT 反推） | 84.5 |  |
| k′p μA/V² |  | 34.8 |  |
| 迁移率比 Mobility ratio |  |  |  |

# 六、注意事项  6  Notes

PMOS 的电压都取绝对值：|VGS|、|VDS|、|ID|。
Use magnitudes for PMOS: |VGS|, |VDS|, |ID|.

求 S 要在亚阈值区（电流远小于 VT 处的电流）里取，进入强反型后斜率变小，算出的 S 会偏大。
Take S well below threshold; above it the slope flattens and S comes out too large.

参考结果：VT 约 0.74（n）/ 0.71（p）V，IDSAT 约 440 / 181 μA/μm，S 约 87 mV/dec（n）。你的结果应与此一致（同一模型），差别大时检查算法。
Reference: VT about 0.74 (n) / 0.71 (p) V, IDSAT about 440 / 181 μA/μm, S about 87 mV/dec (n). Yours should agree (same model); if not, check the algorithm.

# 七、思考题  7  Questions

1. 为什么 PMOS 的饱和电流只有同尺寸 NMOS 的一半左右？这对后面画与非门有什么影响？
Why is the PMOS saturation current only about half that of an NMOS of the same size? What does it mean for the NAND gate in step two?

2. 关态电流随温度怎样变化？电机控制芯片装在发热的电机旁边，这意味着什么？
How does the off current change with temperature? What does that mean for a chip mounted next to a hot motor?

# 八、与实际问题的联系  8  Connection to a real problem

机器人问题：关节电机芯片的驱动能力从哪里来

Robot problem: Where the drive strength of the joint-motor chip comes from

芯片的 PWM 输出要在几纳秒内给 H 桥驱动芯片的输入端（约 5 pF）充放电。(1) 用你提取的 IDSAT，估算最小尺寸 NMOS 把 5 pF 从 1.8 V 放电到一半要多久（参考：每微米约 440 μA）。(2) 要在 2 ns 内完成，输出级 NMOS 至少多宽？(3) 实际芯片的输出由 Tiny Tapeout 的 IO 单元驱动，查一下你的结论和它的量级是否相当。

The chip's PWM output must charge the H-bridge driver input (about 5 pF) within nanoseconds. (1) With your IDSAT, how long does a minimum NMOS take to discharge 5 pF from 1.8 V to half (reference: about 440 μA per μm)? (2) How wide must the output NMOS be to do it in 2 ns? (3) On the real chip the Tiny Tapeout IO cells drive the pins: is your answer of the same order?

请在实验报告“实际问题的建模与求解”一栏中按五步完成：① 实际问题 ② 建立模型（写出简化假设）③ 求解 ④ 用实验或仿真检验 ⑤ 指出模型在哪里失效、怎样修正。

In the report section “Modelling a real problem”, work through five steps: ① the problem ② the model (state your assumptions) ③ the solution ④ a check by experiment or simulation ⑤ where the model fails and how to improve it.

生活中的例子：水龙头：拧开一点点就有细流（亚阈值），拧到一定程度水流突然变大（阈值），再拧水流也大不了多少（饱和）。

Everyday example: A tap: a trickle when barely open (subthreshold), a sudden rush past a point (threshold), little more however far you turn it (saturation).

# 九、评分  9  Grading

按贯通项目评分量规：原理 15、数据 20、分析 20、实际问题 20、规范 10、流片就绪度 15。本步的流片就绪度按下表由检查结果自动给分：

Graded with the capstone rubric: principles 15, data 20, analysis 20, real problem 20, presentation 10, tapeout readiness 15. This step's readiness points come from the check results:

| 检查项 Check | 分值 Points | 条件 Condition |
| --- | --- | --- |
| 检查命令运行成功，参数表完整 Check ran, table complete | 5 | params.json 有 VT、IDSAT、IOFF、S |
| VT 与参考值相差 ≤ 0.02 V VT within 0.02 V of reference | 5 | ¦vt − 参考¦ ≤ 0.02 |
| IDSAT、S 与参考值相差 ≤ 3% IDSAT and S within 3% | 5 | 相对误差 ≤ 3% |